Final Project Proposal (COSC 41001: Advanced NN: Deep Learning for Spatial Analysis)

Nicolas Maitland 100259181

Title: Safety‑First Autonomous AI RC Race Car: Lane Following, Obstacle Handling, and Stability Control on Edge Hardware

Domain & Topic: Autonomous Driving — Indoor autonomous RC race car with lane detection, obstacle avoidance, and basic yaw/slip stabilization.

Team size: Two members (as allowed by the course).

1) Introduction

Overview. We will build a 1/10–1/16 scale RC car that drives itself around an indoor track with taped lanes, using a front‑facing camera and deep neural networks for perception, and lightweight control logic for safe actuation. The system runs fully on‑device (Jetson Nano preferred; Raspberry Pi 4 as backup), demonstrating real‑time edge AI.

Relevance & importance. The project translates core self‑driving concepts—lane keeping, obstacle handling, and stability—into a safe, low‑cost platform. It showcases computer vision + deep learning on embedded hardware, aligning with industry trends (edge deployment, latency‑sensitive inference) and the course emphasis on deep neural networks and OpenCV‑centric tooling.

Primary objective & impact. Deliver a reliable, safety‑first autonomous mini‑vehicle that:

1. Tracks lanes robustly on varied indoor lighting;

2. Detects and responds to obstacles/stop markers;

3. Mitigates slip/drift on a designated “low‑friction” patch;

4. Is packaged for deployment as part of INFT 41000 (edge device or cloud handoff).

2) Project Description

2.1 Scope

Included

• Indoor oval/road course with single or double taped lanes; repeatable layout.

• Perception with deep learning (primary) and light CV safety checks.

• Lane keeping at modest speed; stop/slow for obstacles or colored “stop boxes.”

• Basic slip/yaw monitoring using IMU; throttle/steering moderation on “ice.”

• On‑device inference (Jetson Nano; Pi 4 fallback) and OpenCV‑based pre/post‑processing.

Excluded / out of scope

• Multi‑vehicle racing, V2V comms, GPS/SLAM mapping, high‑speed drifting maneuvers.

• Outdoor operation, dynamic pedestrians, complex traffic rules/signs beyond a “stop box.”

2.2 Objectives (measurable)

• Lap reliability: ≥ 90% lap completion without human intervention on the standard course.

• Lane tracking quality: Mean lateral error ≤ 15 cm on straights; ≤ 25 cm on curves (video‑based measurement).

• Obstacle response: Detect stop marker ≥ 60 cm ahead and stop within 20–40 cm consistently.

• Slip mitigation: On low‑friction patch, detect abnormal yaw in ≤ 150 ms and reduce throttle; no spin‑outs.

• Real‑time performance: End‑to‑end control loop ≥ 10 Hz on device.

2.3 Methodology

A) Overall approach (DL + safety logic)

• Lane keeping (primary DL path): Supervised behavioral cloning CNN that maps camera frames (e.g., 120×160/160×160) to steering. Optional second head or rule ties throttle to steering curvature.

• Obstacle/stop handling (hybrid):

• Classical CV (OpenCV HSV thresholding + morphology in ROI) for a colored stop marker;

• Ultrasonic front sensor as a hard safety brake;

• (Stretch) Lightweight object detector (ONNX/TFLite) for generic obstacles on Jetson.

• Slip/yaw mitigation: IMU monitors yaw rate vs. commanded turn; reduce throttle or gently counter‑steer if deviation exceeds threshold; low‑pass/Kalman smoothing to curb jitter.

B) Development steps

1. Platform bring‑up: RC chassis → servo/ESC control; mount camera (USB/PiCam) and IMU; validate power.

2. Teleop + data logging: Gamepad/web UI to drive the taped course; record (image, steering, throttle, IMU).

3. Model training (Apple Silicon host): Train CNN (PyTorch/TensorFlow), augment (brightness/contrast, flips), validate; export to ONNX.

4. On‑device inference: Load via OpenCV DNN (CUDA on Jetson) or TFLite (Pi); integrate into control loop.

5. Obstacle logic: OpenCV HSV mask in lower‑frame ROI; ultrasonic stop if distance < threshold.

6. Slip logic: Compute expected vs. actual yaw; throttle clamp and short counter‑steer routine; tune thresholds.

7. Tuning & evaluation: Iterate hyper‑parameters, controller gains, ROI sizes; collect metrics/video.

8. Packaging for INFT 41000: Service wrapper (REST/CLI), config, and boot scripts for edge deployment.

C) Tools & technologies (with OpenCV)

• OpenCV (cv2): camera capture, undistortion/calibration, color space ops, morphology, ROI, perspective warp (for debugging), cv2.dnn for ONNX runtime, drawing overlays; cv2.KalmanFilter (optional) for state smoothing.

• DL stack: PyTorch or TensorFlow on Apple M1 Pro for training; export to ONNX → OpenCV DNN / TensorRT (Jetson).

• Embedded: Jetson Nano (JetPack + CUDA/cuDNN/OpenCV), Raspberry Pi 4 (OpenCV + TFLite), pyrealsense2 (if using RealSense D435 depth), IMU via I²C, HC‑SR04 ultrasonic via GPIO.

• MLOps/dev: Python, NumPy, ONNX Runtime/TF‑Lite (as needed), logging & metrics notebooks.

D) Data (acquisition & preprocessing)

• Collection: 10–20 teleop laps in both directions; include recovery maneuvers (edge cases).

• Labels: Steering/throttle captured automatically; IMU synchronized; optional manual tags for “slip events.”

• Pre‑proc: Center crop roadway, resize, normalize; augmentation for lighting/color variance.

• Ethics/privacy: Indoor track with no bystanders; no PII captured.

• Storage: Organized sequences (image + JSON/CSV logs), train/val split (e.g., 80/20).

E) Expected outcomes (what success looks like)

• Live autonomous laps with clear overlay (lane ROI, predicted steering).

• Consistent stop at colored marker and failsafe stop with ultrasonic trigger.

• Throttle moderation on “ice,” observable yaw stabilization behavior.

• Metrics report, ablation (with/without slip logic), demo video, reproducible setup script.

3) Challenges & Limitations

• Real‑time constraints: Pi 4 may cap at ~10 FPS; mitigated via lower resolution/ROI, ONNX/TensorRT on Jetson.

• Generalization: Model may overfit to one lighting/track; mitigated via augmentation and data refresh.

• Sensing noise: Ultrasonic jitter; IMU drift—addressed with filtering and hysteresis thresholds.

• Slip control complexity: True drift control needs advanced vehicle models; we restrict to stability (ESC‑like) behavior.

• Power/thermal: Jetson power draw/heat; use adequate 5V/3A supply and cooling.

• Safety: Always maintain manual kill switch/ESC cutoff and test at low speeds first.

4) Timeline & Milestones

Milestone 0 (Oct 14, 2025): In‑class application explanation & approval.

Week 1 (Oct 15–21) — Bring‑up & track

• Select chassis, mount camera/IMU, wire ESC/servo, verify teleop; lay out taped track; draft test plan.

Week 2 (Oct 22–28) — Data pipeline & baseline

• Teleop logging; OpenCV preview; first CNN architecture; training on Mac; baseline autopilot at low speed.

Week 3 (Oct 29–Nov 4) — On‑device inference

• Export to ONNX; integrate cv2.dnn on Jetson/Pi; achieve ≥10 Hz loop; lap‑tuning.

Week 4 (Nov 5–11) — Obstacle handling

• HSV stop‑marker detection + ultrasonic brake; evaluate stopping distance; tuning & tests.

Week 5 (Nov 12–18) — Stability/slip logic

• IMU fusion; define yaw deviation thresholds; throttle clamp/counter‑steer; test on “ice” patch.

Week 6 (Nov 19–25) — Robustness & metrics

• More data (lighting variants); augment; re‑train; quantitative metrics; video capture.

Week 7 (Nov 26–Dec 2) — Packaging for INFT 41000

• Boot scripts, config, service wrapper (edge); documentation & reproducibility.

Week 8 (Dec 3–9) — Polish & deliver

• Final demo run; ablation results; finalize report & PDF submission to DC Connect.

Team & Roles

• Member A (Embedded & controls): Hardware integration, ESC/servo I/O, ultrasonic/IMU, control loop, safety/kill switch.

• Member B (Perception & ML): Data pipeline, model training on M1 Pro, ONNX export, on‑device inference, metrics.

• Both: Data collection, testing, tuning, documentation, final demo.

Connecting to INFT 41000 (Deployment)

We will deploy the trained model to edge hardware (Jetson Nano preferred) with a service wrapper (systemd or simple CLI) and configuration profiles (track layout, thresholds). For the INFT 41000 deliverable, we will demonstrate:

• On‑device startup (auto‑run at boot),

• Configurable modes (teleop, autonomous, test), and

• Telemetry output (JSON logs, optional Web UI) simulating real‑world IoT/embedded deployment. Cloud handoff (optional) can forward telemetry to a dashboard but inference stays on‑device to meet latency/privacy goals.

(Optional) Bill of Materials (indicative)

• RC chassis (1/10–1/16) with ESC + steering servo

• NVIDIA Jetson Nano (or Raspberry Pi 4), microSD, 5V/3A–5A supply

• USB camera or Pi Camera; (optional) Intel RealSense D435

• IMU (MPU‑6050/9250), HC‑SR04 ultrasonic + wiring

• Mounts, tape for lanes, foamboard base, safety switch