## IMU Integration and New Autonomous Features

In continuation of the original project plan, this section introduces new IMU-enhanced functionalities made possible by the Intel RealSense D435i’s built-in Bosch BMI055 IMU. The D435i adds a 6-DoF inertial sensor (3-axis gyroscope + 3-axis accelerometer) to the depth camera, providing valuable motion and orientation dataedge-ai-vision.com. These IMU-based features will augment the existing camera-driven autonomy with improved stability, safety, and environmental awareness. The RealSense SDK 2.0 supports streaming the IMU data alongside RGB-D framesedge-ai-vision.com, allowing synchronization via timestamps. The BMI055 gyroscope can sample at 200–400 Hz and the accelerometer at 62.5–250 Hz (±4g range for accel, ±1000 °/s for gyro), with timestamps accurate to 50 μs for alignment. Running these additional IMU streams and algorithms on the Jetson Nano is feasible with minimal overhead – the data rates (a few hundred Hz) and computations (simple filters/control logic) are lightweight compared to deep learning inference on images. Below, we detail each new IMU-enabled feature, including the role of the gyroscope vs accelerometer, integration with the RealSense pipeline, any required filtering or sensor fusion, real-time performance considerationson Jetson Nano, and how the feature enhances the camera-based control loop. We also outline logging/visualizationplans for IMU data and a testing strategy for each behavior to ensure alignment with project goals and the final project rubric.

### IMU-Based Drift Detection and Correction (Stability Control)

Objective: Detect and automatically correct for vehicle drift or oversteer during aggressive turns using IMU data. This feature acts like an electronic stability control or RC gyro for the car, intervening to counteract excessive yaw rotation (spin-out) by modulating steering and throttle.

Sensor Cues: The gyroscope’s yaw rate is the primary indicator of a drift. When the car starts to oversteer, the yaw angular velocity will spike beyond what the steering input or intended path dictates. The accelerometer can provide secondary cues – for example, a high lateral acceleration without a corresponding turning radius (or a sudden drop in longitudinal acceleration if the car is sliding) – but the gyroscopic yaw-rate is the most direct way to sense unintended rotation. The BMI055 gyro, sampling at up to 400 Hz, will capture rapid fishtailing or rotation on the z-axis (yaw) in real time.

Integration & Logic: The IMU stream will be processed in parallel with the camera frames. We will implement a yaw-rate feedback loop that monitors the gyro data every few milliseconds. If a sudden yaw acceleration is detected that exceeds a threshold (tuned via testing) and is not commensurate with the commanded steering angle, the system will trigger a corrective response. Specifically, the steering servo will be automatically adjusted in the opposite direction (“counter-steering”) to oppose the unintended spin, and/or the throttle will be momentarily reduced to allow the tires to regain traction. This mimics how an RC drift-assist gyro stabilizes a car: the gyro reading of yaw is used to rapidly counter-steer the wheels to correct the slipscispace.com. For example, if the car’s rear begins to slide out to the right during a left turn (yaw rate too high to the left), the controller will briefly steer right to counteract the drift, then smoothly return to the intended path once stability is regained. The accelerometer may also be used to detect if the car is not actually turning as expected (e.g. wheels spinning with little lateral acceleration, indicating a slide) and then cut throttle. All IMU data handling will use the RealSense API’s callbacks to get synchronized timestamps, ensuring we correlate the drift event with the current vehicle state.

Filtering and Stability: To avoid false positives (e.g. steering rapidly but intentionally), we will apply a small amount of filtering to the yaw-rate signal. For instance, a short moving average or low-pass filter can smooth out high-frequency noise from vibrations. We will also set a deadband threshold so that minor yaw fluctuations do not trigger the system. The control logic itself can act as a high-pass filter – only rapid, excessive yaw changes (beyond what the current steering command should produce) will invoke correction. Because the gyro has minimal drift over the short intervals of detection, a complex Kalman filter may not be required; a simple thresholding algorithm with a bit of smoothing is sufficient for real-time stability control. The key is a fast response: at 200–400 Hz IMU updates, the Jetson Nano can compute corrective commands with only a few milliseconds latency, which is crucial for counter-steering effectively during a slide.

Edge Processing Feasibility: Implementing drift correction is well within the Jetson Nano’s capabilities. Reading the IMU at 200+ Hz and performing a few arithmetic operations and conditional checks is trivial (on the order of <1% CPU). The IMU data is low-bandwidth (a few hundred bytes per second), so it will not stress the USB 3.0 interface or the CPU. The control output (steering/throttle adjustments) can be sent to the Arduino/motor controller at a similar high rate. The overall latency from sensing a yaw spike to issuing a counter-steer command is expected to be only ~5–10 ms, which is negligible for the scale of an RC car (which has dynamics on the order of hundreds of ms). Thus, the feature should run reliably in parallel with the main camera inference loop without dropping frames.

Enhancement to Camera Loop: This drift correction layer greatly augments the camera-based driving. The vision model or autonomy code might command a certain turn, but if the car loses traction or over-rotates, the camera alone might not immediately recognize the instability (until perhaps the car is far off the intended path or the view changes drastically). The IMU provides an instantaneous measure of the car’s actual rotation, allowing us to clamp down on physical instability before it manifests in the camera’s view. In essence, the IMU adds a fast inner-loop control for stability, while the camera remains the outer-loop for path planning. This will result in a car that corners more predictably and is forgiving of slippery surfaces or sudden maneuvers, rather than spinning out – improving robustness of autonomy.

Logging and UI: We will log events of drift intervention, including timestamp, yaw rate, and the corrective action taken. A debugging interface (e.g. a simple console print or on-screen message) can indicate “Stability Control ACTIVE” when a drift correction triggers. Additionally, we may plot the gyro yaw-rate vs. time and mark where interventions occur, to fine-tune the threshold. This logging will help during development to adjust sensitivity – ensuring it triggers on real oversteer events but not on normal sharp turns.

Testing & Evaluation: Each drift-correction behavior will be rigorously tested:

- Controlled Slip Test: On a low-friction surface (e.g. smooth concrete or dusty floor), instruct the car to make a sharp turn at speed to intentionally induce oversteer. Verify that the IMU-based system detects the high yaw and automatically counter-steers or cuts power, and observe if the car recovers alignment instead of spinning out. We can compare runs with and without the feature: with drift control, the car should maintain a tighter turning radius and require less manual correction.

- Quantitative Metrics: Use logged data to measure peak yaw rates during turns with vs. without the correction active. A successful system should show a reduced peak yaw (preventing it from exceeding safe values) and quicker damping of oscillations. If available, an overhead video or motion capture of the car’s trajectory could confirm that it stays more stable (less sideways sliding).

- Parameter Tuning: Through testing, adjust the yaw-rate threshold and counter-steer gain. For example, start with small steering corrections and increase until the car just avoids spinning. Ensure the threshold avoids false triggers (the car should not fight the driver/model during intended aggressive but controlled turns). We’ll also test quick S-turn maneuvers to ensure the system doesn’t falsely interpret rapid but controlled alternate turns as a drift – the filtering and threshold should handle this.

- Fail-safe: Intentionally push beyond the system’s capability (e.g. a very fast turn on a very slick surface) to see how it behaves. The car might still spin if physics demands, but it should at least cut throttle to minimize chaos. This ensures the feature degrades safely in extreme cases. All these tests will be documented, addressing rubric points on validation and safety.

### IMU-Aided Steering Smoothing via Sensor Fusion

Objective: Improve the stability of steering commands by fusing camera-based predictions with IMU data to dampen jitter or oscillation. This feature will prevent rapid left-right steering changes (“twitchiness”) that can occur due to noisy vision outputs or rapid model inference fluctuations, by using the IMU to inject a sense of physical stability into the steering control.

Sensor Cues: The key IMU signal here is the gyroscope’s measurement of angular velocity (yaw rate), which tells us how quickly the car is actually turning. The accelerometer could also contribute indirectly by indicating if the car is in a steady state or undergoing lateral acceleration, but the gyro is most directly useful. If the vision model commands a quick left-right-left sequence but the gyro shows that the car’s heading hasn’t changed correspondingly (i.e. the car can’t physically match that oscillation so fast), we know those rapid flips are likely spurious or should be moderated. Conversely, if the car is already mid-turn (non-zero yaw rate in one direction), the steering commands should not suddenly reverse unless necessary – inertia would make the car resist instant reversal.

Integration & Fusion Approach: We will implement a form of sensor fusion or filtering that combines the vision-predicted steering angle with the IMU’s observed rotational movement to produce a smoother steering command. One approach is using a complementary filter concept: treat the vision model’s steering suggestion as a potentially noisy high-level command, and treat the gyro (integrated to an angle or used as rate feedback) as the stabilizing factor. For example, if the camera inference suddenly oscillates, the system can weight the current actual trajectory more strongly to avoid sharp jerks. In practice, this might mean applying an exponential smoothing or low-pass filter to the steering commands where the filter’s responsiveness is tied to the IMU readings. When the gyro indicates the car is stable (low yaw-rate), we clamp rapid changes in steering – effectively filtering out quick swings that the car hasn’t responded to yet. If the gyro indicates the car is already turning or the rate of rotation is increasing, we allow the steering to change direction but in a controlled manner rather than instantly.

Concretely, we can maintain a filtered steering command S_f. Each control cycle (e.g. at camera frame rate ~30 Hz), update S_f as:

- If the new vision-predicted steering S_v differs drastically from the current S_f, check the yaw-rate ω. If |ω| is small (car not turning much), it suggests S_v might be a flicker; we then moderate the change by blending: S_f = S_f + α*(S_v - S_f), where α is a small factor (slowly move toward the new command). If |ω| is high or increasing in the opposite direction of the commanded change, likewise we smooth it to avoid destabilizing the ongoing turn.

- If the vision change is in line with the car’s momentum (e.g. model corrects by steering slightly more into a turn and ω indicates we are turning that way), we can allow a faster update (larger α) since it won’t cause a harsh reversal. Essentially, we’re using yaw feedback to impose a rate limit on steering changes that contradict the vehicle’s current motion.

This can also be seen as adding a derivative feedback to steering: using yaw-rate as a damping term. Many control systems use gyro feedback to damp oscillations – here the effect is similar, preventing overshoot. The RealSense IMU data will be read synchronously with frames (or even at a higher rate in its own thread) to ensure we have up-to-date rotation info for each steering update.

Filtering and Sensor Fusion: A complementary filter or simple PID-like approach will be applied. We avoid heavy computation by using a heuristic sensor fusion rather than a full Kalman filter, which is overkill for a one-dimensional steering signal. The complementary filter idea is apt: gyros excel at short-term smooth motion sensing but can drift, whereas “vision” (or the high-level desired direction) provides long-term reference but can be noisydev.realsenseai.com. By blending them, we get a stable but responsive steering signal. For instance, we might set a parameter that 90% of the steering command comes from the previous filtered command adjusted by gyro feedback, and 10% comes from the new vision suggestion – this ratio can be tuned (akin to the α in complementary filter theory, where high α favors gyro stability and low α favors new input)dev.realsenseai.com. The result is that momentary swings in S_vare largely ignored unless corroborated by a change in vehicle orientation. If the vision system consistently demands a turn, the filtered command will eventually follow (accelerometer and gyro will also reflect the car beginning to turn, so we’ll allow it). We will tune the filter weight to remove jitter while adding minimal lag. The accelerometer can also be used to detect the initiation of a turn (lateral acceleration) as another cue to trust the change, but primarily the gyro yaw-rate suffices.

Performance on Jetson Nano: This sensor fusion step involves a few arithmetic operations per frame (and potentially some small matrix if extended, but essentially negligible). The IMU data reading and combining can easily run at the camera frame rate or higher. The latency added is minimal – on the order of a few milliseconds for filtering math. The Jetson’s CPU can handle this continuously alongside other threads. If needed, we can run the filtering in a separate thread triggered by IMU callbacks (since IMU frames are asynchronous), updating a shared steering value with proper locking (following a scheme similar to Intel’s IMU example that uses a mutex for shared orientation datadev.realsenseai.comdev.realsenseai.com). This ensures up-to-date orientation info is always considered without slowing the main loop. The overall effect should be a smoother control with no perceivable delay to the vehicle’s reaction time, as the damping is only removing unrealistic high-frequency components.

Enhancement to Camera Loop: Camera-only steering control can be susceptible to over-correction – the model might zigzag if it perceives slight deviations or if the image input is noisy. By adding IMU-based smoothing, we effectively filter out control noise, leading to a smoother trajectory. This not only improves ride quality but can also help the model itself (a car that oscillates less will generate more stable camera imagery, reducing feedback instability). In sum, the IMU fusion acts as a stabilizer for the vision system, ensuring that steering changes reflect actual needed adjustments rather than sensor noise or model uncertainty. This addresses potential rubric points on control refinement and innovation: we are introducing a form of closed-loop feedback where previously it might have been open-loop from vision to actuators.

Logging and Visualization: During testing, we will log both the raw vision-predicted steering commands and the final IMU-smoothed commands. Plotting these over time (vs. vehicle yaw) can illustrate how the filter behaves (e.g. oscillations in the raw command that are absent in the filtered command). This will help in explaining and justifying the effectiveness. We can also display a real-time debug value for current yaw-rate and the damping effect (perhaps print out if a command change was limited by the smoothing algorithm for transparency). This visualization is useful to confirm that we are not over-smoothing (which could cause lag in responsiveness). If possible, an on-screen overlay showing a “steering gauge” for commanded vs. actual steering might be implemented for demonstration.

Testing & Evaluation: We will evaluate steering smoothing in various scenarios:

- Zigzag Course Test: Set up a slalom or gentle S-curve course. Run the car with the vision model controlling steering without smoothing, and record its path (likely the car might overshoot or oscillate around the center). Then enable the IMU smoothing and repeat. We expect to see the car follow the intended path with fewer oscillations. Quantitatively, we can measure the variance in steering angle over time and the deviation from the centerline; the smoothed control should show reduced overshoot and smoother transitions.

- Noisy Input Simulation: If possible, intentionally inject noise into the model’s steering output (or use a high-frequency dither on the manual steering input during testing) to simulate an erratic model. Verify that the IMU fusion damps the high-frequency component. We can measure frequency content of the steering signal before and after – the filtered output should have less high-frequency content (which corresponds to jitter).

- Response Timing: Ensure the smoothing does not introduce too much lag: e.g. a sudden necessary avoidance maneuver should still happen in time. We test this by having a quick required turn (like a sudden obstacle or sharp corner). The car should still make the turn, proving we haven’t made it overly sluggish. Tuning α is crucial here – and this test ensures our chosen value balances smoothness with responsiveness (trade-off noted in complementary filtersdev.realsenseai.com).

- User/Model Feedback: If available, test with the actual autonomy model in a realistic setting (e.g. following a lane or wall). Subjectively assess if the car’s steering looks more natural and stable. Collect any telemetry from the model (like how often it has to correct back and forth) – the smoothing should reduce rapid oscillatory corrections, addressing rubric criteria on performance improvement. All these test results will be documented, demonstrating improved stability due to sensor fusion.

### IMU-Based Crash and Rollover Detection (Safety System)

Objective: Implement a safety mechanism to detect if the vehicle has crashed, collided, or flipped over, and respond by cutting off throttle and alerting the user/logs. This feature uses the IMU as a crash sensor – similar to how phones or drones detect hard impacts – to protect the hardware and bystanders, fulfilling a safety requirement often valued in project rubrics.

Sensor Cues: A crash or rollover event produces distinctive IMU signatures:

- A collision (e.g. hitting a wall or obstacle) results in an abrupt spike in acceleration. The accelerometer reading in one or more axes will jump suddenly, far exceeding normal driving levels. For example, a frontal crash might show a sharp spike in +X acceleration (if X axis is forward) as the car decelerates extremely quickly. The BMI055 can measure up to 4g; a crash could momentarily approach or exceed this (potentially clipping). We will monitor the magnitude of the acceleration vector (√(x²+y²+z²)) to catch large impulses regardless of direction.

- A rollover or flip will manifest as an unusual and sustained change in orientation: the pitch or roll angle of the IMU will go beyond normal range (e.g. suddenly > 60° or even 90° from upright). The gyro might also register a high rotation rate during the flip. Even if the impact is not strong, the fact that the car’s orientation is now on its side or inverted can be detected from the gravity direction measured by the accelerometer.

Integration & Logic: The IMU data will be continuously monitored in a background task for these extreme events. We will implement two detection strategies in parallel:

- Accelerometer Spike Detection: We set an acceleration threshold that significantly exceeds any values seen during regular driving (including bumps). For example, if normal driving yields <1g peaks (even sharp turns or braking might be around 0.5–0.8g), we might set a threshold around e.g. 2g. If the acceleration vector magnitude exceeds this threshold within a very short time window, we interpret it as a crash impact. To avoid noise triggering it, we may apply a slight low-pass filter or take a maximum over a few milliseconds. Research suggests smoothing the accel data (e.g. filtering at very low frequency) and then thresholding the peak can reduce false positivesleotac.github.io. We will likely implement a simple approach: compute a rolling average of the acceleration magnitude or use a one-pole low-pass filter to get the general trend, then if that smoothed value jumps above threshold, it’s a crash eventleotac.github.io. The threshold and filter constant will be tuned by dropping the car from small heights or running into soft objects to calibrate.

- Tilt (Roll/Pitch) Detection: Continuously estimate the tilt of the vehicle. Using accelerometer data, we derive pitch and roll angles. If the car’s pitch or roll goes beyond a “possible normal” limit (for instance, > 75° roll meaning it’s almost on its side or upside down), we classify it as a rollover. A simpler check is if the gravity direction flips (e.g. Z-axis acceleration which is ~+1g when upright becomes near 0 or negative), indicating an upside-down orientation. This check might even catch a gentle tip-over that doesn’t produce a big acceleration spike.

When either condition triggers, the system will immediately cut off throttle output. On the Jetson Nano, this means sending a command to the motor controller (via Arduino or ESC) to set speed = 0 (and possibly brake). We will design it such that normal operation won’t resume until a manual reset or a certain procedure (to avoid the car suddenly driving off after a crash if the model still outputs throttle). In addition, the event will be logged and an alert generated (e.g. an on-screen message “CRASH DETECTED – Motors stopped” or an LED indicator on the car). If the car has a UI or is monitored via a dashboard, we’ll ensure the message is relayed.

Filtering and Considerations: As mentioned, filtering is crucial to avoid false triggers:

- For acceleration, momentary bumps or jumps should ideally not immediately cut throttle unless truly severe. By using a very short low-pass filter or requiring the spike to last for a few milliseconds, we can filter out high-frequency vibration. We might also incorporate the gyro: a crash often coincides with a sudden change in velocity (accel spike) followed by chaotic rotations – if we see both a spike and a concurrent unusual gyro reading, that increases confidence it’s a crash, though this is optional refinement.

- For tilt, we will use the complementary filter orientation estimate (see next section on incline sensing) to have a smooth pitch/roll value. This prevents noisy accelerometer readings (e.g. from driving on rough ground) from falsely indicating large tilts. The complementary filter will ensure we rely on gyro integration for short term stability and accelerometer for long-term angle, giving an accurate orientationdev.realsenseai.com. Only if that orientation is far outside normal (car will never intentionally pitch >45° in usage) do we trigger rollover. We will incorporate a small delay so that, for example, if the car hits a ramp at 45° briefly, it doesn’t cut power thinking it’s flipped – the system could require the tilt to persist for say >0.5 second or the car to actually invert (near 180° pitch/roll) to trigger.

Edge Performance: The crash detection runs in real-time but its computational load is minimal. Checking a threshold and filter on a 200 Hz data stream is trivial for the CPU. The main requirement is reliability and low-latency response – which the Jetson can handle easily (the throttle cut command can be issued within <10 ms of impact detection). The only hardware consideration is that the BMI055’s ±4g range may saturate for extremely hard crashes; however, if we saturate the accelerometer, it’s almost certainly a crash-level event anyway. We just ensure our logic handles saturation as a trigger. The Jetson’s I/O will promptly send the stop signal; any slight USB jitter is inconsequential relative to the physical event (crash dynamics are slower than our detection by orders of magnitude).

Enhancement to System: This safety feature doesn’t directly aid driving performance, but it is crucial for preventing damage and enabling quicker recovery. In an autonomous run, if the car crashes without detection, it might keep trying to drive (spinning wheels, possibly causing more damage or confusion to the vision system). With IMU crash detection, the system transitions to a safe state immediately. This aligns with good engineering practice and likely addresses rubric criteria regarding safety considerations, fault handling, and system robustness. It also provides valuable feedback for improving the autonomy – the log of crash events (timestamp and possible cause) can help diagnose what went wrong (e.g. an obstacle not detected by vision but indicated by a sudden stop). In summary, it augments the project by adding a layer of fault tolerance and safety that pure vision could not provide.

Logging and UI: Every crash or flip event will be logged with a timestamp and the sensor readings (e.g. peak acceleration, angle). We will have a flag in the system status that can be displayed: for instance, a red status light in a GUI that turns on when a crash is detected (and possibly remains on until reset). If the Jetson is connected to a monitor or SSH console, a printout like “EMERGENCY STOP: Crash detected at 12:05:27 (accel=3.5g)” will be output. This not only alerts the user but provides a data point for later analysis (did we trigger appropriately, was it a false alarm, etc.). For rollover, similarly we can output the angle detected. We might even incorporate a buzzer or LED on the physical car to signify the system halted due to a crash (useful in testing scenarios where the car might be out of immediate sight).

Testing & Evaluation: We will test crash/rollover detection in safe, controlled ways:

- Bench Drop Test: With the car powered (wheels off ground), drop it a short distance onto a soft surface (or simulate a jolt) to generate a spike. Verify the system triggers and cuts throttle. Increase drop height gradually to see the threshold in effect. We expect no trigger for very small jostles, and a trigger for larger shocks.

- Impact Test: Gently drive the car into a lightweight obstacle (e.g. a foam block or cardboard box) at moderate speed. This creates a deceleration spike without harming the car. Check that throttle is cut off at impact. Ensure the log records the event. Also, confirm that normal driving over bumps (e.g. a small ramp or rough terrain) does notfalsely trigger a crash alert – those acceleration profiles are usually more prolonged and lower magnitude, which our filter/threshold should ignore.

- Rollover Test: Manually flip the car upside down while it’s running (or tilt it slowly beyond 90°) to simulate a rollover. The system should detect the abnormal orientation and cut power. We will also test partial flips: tilt to ~60° and return to see that it does not trigger unless truly past the limit. If the car is capable of driving on a slope, we’ll drive it on a moderate incline to ensure that doesn’t set off the rollover detect (the incline awareness feature will handle that scenario separately).

- Recovery Procedure: After a triggered event, verify that the system remains in a safe stopped state (motors off) until we intentionally reset it. This is important so that after, say, picking up the car to set it upright, it doesn’t lurch forward unexpectedly. We’ll document the required reset step (maybe a specific controller input or restart).

- Data Verification: Review the logged sensor data around the event to ensure it matches expectations (e.g. “crash detected” log shows a spike of X g which correlates with what we know happened). Fine-tune the threshold if needed to avoid any near-miss (for instance, if a slightly rough driving triggered it erroneously, we might raise threshold or adjust filter).

Overall, this testing ensures the crash detection is reliable and meets any rubric requirements for demonstrating safety and testing edge conditions.

### IMU-Enhanced Incline Awareness and Terrain Sensing

Objective: Leverage the IMU’s ability to sense orientation (gravity vector) to make the robot aware of inclines, declines, and uneven terrain. This has two major benefits: (1) Adjusting vehicle control on slopes (e.g. throttle modulation to maintain safe speed uphill vs downhill), and (2) Improving environmental perception by compensating for camera tilt, thereby avoiding false obstacle detections or navigation errors caused by the camera being pitched on a slope.

Sensor Cues: The accelerometer in the IMU constantly measures gravity along with motion. By observing the accelerometer readings at rest or low acceleration, we can derive the pitch and roll angles of the car relative to the horizontal plane. For example, if the car points uphill, the gravity vector will tilt backward relative to the IMU axes. The gyro also contributes by measuring rotational changes – using sensor fusion, we maintain an ongoing estimate of the car’s orientation. In practice, we will compute the gravity vector direction from accelerometer data and use that to find the tilt: pitch (rotation around the Y-axis, fore-aft tilt) and roll (rotation around X-axis, side tilt). Yaw (heading) is not directly needed for incline awareness, since the concern is the angle relative to gravity.

Orientation Estimation (Filtering): To get a stable reading of pitch/roll even when the car is moving, we will use an IMU fusion algorithm (like a complementary filter or a simplified AHRS filter) combining gyro and accelerometer. The complementary filter will weight the accelerometer readings (which give an absolute reference to gravity, but noisy under acceleration) with the gyro integration (which is smooth and responsive, but drifts)dev.realsenseai.com. Intel’s sample implementation for D435i uses a complementary filter with a parameter α ~ 0.98 (98% gyro, 2% accel) for orientationdev.realsenseai.com, which we can adopt or tune similarly. This will run continuously at, say, 200 Hz, updating pitch and roll angles every IMU frame. The result is a real-time estimate of the car’s attitude that is robust to bumps and vibrations. (If the car has prolonged acceleration, the filter will rely on gyro short-term, then once steady, accel will correct any drift, ensuring we always converge back to the true gravity direction.)

Incline Detection and Throttle Control: With a reliable pitch angle (θ_pitch):

- We can detect uphill vs downhill: a positive pitch (nose up) indicates climbing, negative pitch (nose down) indicates descending. The magnitude tells the steepness. When descending (beyond a small threshold, e.g. >5° downhill), the system will automatically reduce throttle to prevent the car from gaining too much speed due to gravity. This is analogous to engine braking or downhill speed control in cars. For example, if the car is in autonomous mode moving at a set speed, on a downhill we might scale back the motor command to maintain that target speed, rather than letting gravity accelerate it. On an uphill, if the objective is just to maintain control, we might not necessarily increase throttle (that could spin wheels if too steep), but we will be aware that the car might slow down and ensure any following distance logic accounts for it. In more advanced implementation, one could slightly boost power uphill to maintain speed, but our primary concern is the downhill case (to avoid runaway).

- If the car has different driving modes, we could even prohibit high-speed mode when a steep downhill is detected, for safety. This will be calibrated depending on the car’s capability and project requirements.

Terrain Sensing and Camera Tilt Compensation: The depth camera’s perception can be skewed by tilt:

- On uphill: The camera (forward-facing) ends up looking more towards the sky; distant ground may appear “far away” and it might miss close obstacles or drop-offs. Knowing the pitch, we could, for instance, adjust the region of interest in the depth frame or angle of view considered. We might also anticipate that the effective range of obstacle detection uphill is reduced (because the camera points up), and thus drive more cautiously.

- On downhill: The camera faces down more, meaning the ground directly ahead appears closer in front of the camera (potentially the depth camera might see the ground at a very short distance and mistakenly treat it as an obstacle). By knowing the pitch angle, we can correct for this geometrically. For instance, if we know the car is pitched down 10°, we can infer that the depth image’s “horizon” is lower; points that appear at 2 m in front might actually be ground at some distance. A simple approach is to use pitch to adjust the criteria for obstacle detection: when pitched forward, require a larger height difference for something to be considered an obstacle (since the ground itself will have a certain slope in view). We can also transform the point cloud or depth data into a level frame using the IMU orientation, then perform ground plane fitting or obstacle detection in that level frame for better accuracy. Essentially, the IMU data can be used to perform a tilt compensation on the depth data, filtering out the ground plane even on slopes.

This integration will likely involve math with the rotation matrix obtained from IMU. The RealSense coordinate system and IMU orientation will be aligned so we can map accelerometer readings to the camera’s tilt. We’ll make sure to calibrate any constant offset (if the camera is mounted at an angle on the car, that bias can be added to the IMU reading).

Edge Processing: Computing orientation and adjusting throttle/decision logic based on it is light-weight. The complementary filter orientation update (some multiplies, adds, and atan2 calls for gravity) is easily done in real-time (even a 1000 Hz loop is fine on Jetson CPU, and we only need ~200 Hz). Adjusting throttle is just a quick scaling operation. The only somewhat heavier part could be transforming depth points for obstacle detection if we do that – but even that is manageable (a matrix multiplication for each point or using the SDK’s pointcloud and applying a rotation). Given the Jetson Nano’s limitations, we will prioritize simple methods: e.g. not full point cloud transformations every frame, but perhaps basic geometric reasoning. For example, we might not literally rotate all points, but use the pitch angle to analytically compute how far ahead the ground will intersect the camera view. This avoids heavy computation and still corrects for tilt.

Enhancement to Autonomy: Incline awareness adds a layer of contextual intelligence to the robot’s decision making that pure vision might lack. It improves safety by controlling speed on hills (preventing crashes due to excessive downhill speed), and it improves perception reliability by accounting for how the camera is oriented. For instance, a vision algorithm might misclassify the ground as an obstacle when the car is tilted – with IMU input, we can prevent false positives, leading to smoother navigation and fewer unnecessary stops or route changes. It effectively makes the depth sensing more robust: the IMU provides an “extra data point” for understanding the 3D world, which is exactly one of the benefits touted for the D435i in roboticsedge-ai-vision.comedge-ai-vision.com. By incorporating the gravity vector, we ensure the car knows which way is truly down, so it can distinguish a real object vs. just an upward slope of the floor, etc. This will likely score well in any rubric aspect related to sensor fusion or innovative use of sensors to solve practical issues.

User Interface and Logging: We will display the current incline angle (pitch) and perhaps roll on the debugging UI. For example, a tilt indicator or simple text “Pitch: +10° (uphill)” can be shown. This not only helps in debugging but also demonstrates the feature during demos. Additionally, when automatic throttle adjustment occurs, we can log that event (e.g. “Downhill detected, limiting speed to X”). If using a remote interface, one could imagine a small icon indicating uphill/downhill mode engagement. All these are optional, but useful for development feedback and for showing the project polish (meeting the rubric’s expectation of good feedback and control).

Testing & Evaluation: We will validate incline awareness and its effects through several tests:

- Static Incline Measurement: Place the car at known tilt angles (e.g. use a ramp or propping up the car at 10°, 20°) and compare the IMU-reported pitch to the expected value (using a digital protractor or the ramp’s known angle). We aim for the orientation estimation to be accurate within a couple of degrees. This tests the sensor fusion calibration.

- Driving on Ramp: Construct a gentle ramp (for example, 15° slope) and have the car drive up and down under autonomous control. Going downhill, observe if the throttle is being reduced. We can measure the car’s speed with and without the feature – without it, the car may accelerate or lurch downward; with it, the speed should remain more constant and controlled. Uphill, ensure the car can still climb (if it slows too much, we might allow a bit more throttle, within safe current limits).

- Obstacle Detection on Slope: Set an “obstacle” (like a shoebox) on a slope and see if the car’s vision system correctly identifies it. Without IMU compensation, the tilt might cause either the ground to look like an obstacle or the obstacle to blend with ground. With the IMU-informed logic, we expect fewer false readings. One way to test: put the car on a down-slope facing an empty floor (no real obstacle) – verify that it does not report a phantom obstacle due to the floor appearing in front in the depth map. The IMU tilt info should help the algorithm recognize that as just ground. Similarly, test on an uphill facing the sky or distant floor – ensure the car doesn’t drive too fast or misinterpret the lack of close depth due to angle.

- Roll Stability: If the car drives over uneven ground where one side is elevated (testing roll), ensure the system doesn’t erroneously cut throttle thinking it’s a rollover. It shouldn’t, because those angles will be moderate (roll maybe 10–20°). But this test is to validate the separation between incline handling and crash handling: e.g. driving on a side slope shouldn’t trigger the crash rollover logic since that requires a much greater angle and likely an impact.

- Integration Testing: Finally, test a full scenario: an autonomous run that includes flat ground, uphill, downhill, and maybe some obstacles. The car should seamlessly adjust speed on the slope, and its obstacle avoidance should not be thrown off by the tilt. We will monitor battery and performance on Jetson Nano during this, confirming that adding the IMU-based computations did not cause frame drops or significant latency in the main loop. This end-to-end test will demonstrate the effectiveness of sensor fusion in improving the robot’s adaptability to real-world terrain.

By thoroughly testing each of the above IMU-enhanced features and integrating their logging/feedback into the system, the updated project plan ensures a robust, safety-conscious, and high-performance autonomous RC car. Each addition aligns with the project goals and evaluation rubric by addressing potential failure modes (drifting, crashes, terrain changes) and leveraging advanced sensor fusion on the Jetson Nano edge platform. The result will be a more resilient autonomous vehicle that intelligently uses all available sensor data (vision + IMU) to navigate and can be convincingly demonstrated with comprehensive metrics and logs to validate its behavioredge-ai-vision.comedge-ai-vision.com. All new capabilities build on the existing camera-based inference loop, enhancing it without fundamentally altering the original architecture – thereby staying true to the initial design while substantially improving functionality in line with final project expectations.

Sources: The implementation draws on sensor specs and best practices from the D435i datasheet, known techniques for drift control in RC carsscispace.com, complementary filtering for IMU integrationdev.realsenseai.comdev.realsenseai.com, and RealSense IMU usage examplesedge-ai-vision.comedge-ai-vision.com, ensuring the design is grounded in proven approaches and data. All testing procedures correspond to the above features and will be documented as evidence of meeting performance and safety criteria.